# NAND-Based Digital Logic Library

- FPGA target: Spartan-7, xc7s50csga324-1

- Tool: AMD Vivado 2025.2

## Gate 1 — NAND Gate

### 1. Objective

2-input NAND gate using

### 2. Boolean Equation

Y=A⋅B

### 3. Verilog RTL — nand_gate.v

module nand_gate(

input a,

input b,

output y

);

assign y = ~(a & b);

endmodule

### 4. Testbench — tb_logic_gates.v

`timescale 1ns / 1ps

module tb_logic_gates;

reg a;

reg b;

wire y;

nand_gate dut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 0; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

$finish;

end

endmodule

### 5. Truth Table

WAVEFORM:

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT2 with input buffers (IBUF) and output buffer (OBUF)

- LUT initialization value: 4'h7

The LUT initialization value encodes the NAND truth table under the standard LUT input convention.

## Gate 2 — NOT Gate (Using NAND)

### 1. Objective

To design and verify a NOT gate by instantiating the previously designed 2-input NAND gate using Verilog structural modeling

### 2. Boolean Equation

Y = NOT(A)

Y = A'

### 3. Verilog RTL — not_gate.v

module not_gate(

input a,

output y

);

nand_gate n1 (

.a(a),

.b(a),

.y(y)

);

endmodule

### 4. Testbench — tb_not_gate.v

`timescale 1ns / 1ps

module tb_not_gate;

reg a;

wire y;

not_gate dut (

.a(a),

.y(y)

);

initial begin

a = 0; #10;

$display("a=%b y=%b", a, y);

a = 1; #10;

$display("a=%b y=%b", a, y);

$finish;

end

endmodule

### 5. Truth Table

### WAVEFORM

Tcl Console output:

a=0 y=1

a=1 y=0

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT1 with input buffer (IBUF) and output buffer (OBUF).

- LUT initialization value: 2'h1.

The LUT initialization value 2'h1 corresponds to the binary configuration 2'b01. It produces an output of 1 for input 0 and an output of 0 for input 1, implementing the NOT function.

## Gate 3 — AND Gate Using NAND + NOT

### 1. Objective

To design and verify a 2-input AND gate by instantiating the previously designed NAND and NOT gates using Verilog structural modeling.

### 2. Boolean Equation

Y = A AND B

Y = A · B

### 3. Verilog RTL — and_gate.v

module and_gate(

input a,

input b,

output y

);

wire n;

nand_gate n1 (

.a(a),

.b(b),

.y(n)

);

not_gate n2 (

.a(n),

.y(y)

);

endmodule

### 4. Testbench — tb_and_gate.v

`timescale 1ns / 1ps

module tb_and_gate;

reg a;

reg b;

wire y;

and_gate dut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 0; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

$finish;

end

endmodule

### 5. Truth Table

### WAVEFORM

Tcl Console output:

a=0 b=0 y=0

a=0 b=1 y=0

a=1 b=0 y=0

a=1 b=1 y=1

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT2 with input buffer (IBUF) and output buffer (OBUF) is the expected implementation on the selected Spartan-7 FPGA, provided synthesis optimizes the logic into one LUT.

- LUT initialization value: INIT: 4'h8 (as observed).

The LUT initialization value 4'h8 corresponds to the binary configuration 4'b1000. For a two-input LUT, this configuration produces an output of 1 only when both inputs are 1, implementing the AND function.

## Gate 4 — OR Gate Using NOT + NAND

### 1. Objective

To design and verify a 2-input OR gate by instantiating the previously designed NOT and NAND gates using Verilog structural modeling.

### 2. Boolean Equation

Y = A OR B

Y = A + B

### 3. Verilog RTL — or_gate.v

module or_gate(

input a,

input b,

output y

);

wire na;

wire nb;

not_gate n1 (

.a(a),

.y(na)

);

not_gate n2 (

.a(b),

.y(nb)

);

nand_gate n3 (

.a(na),

.b(nb),

.y(y)

);

endmodule

### 4. Testbench — tb_or_gate.v

`timescale 1ns / 1ps

module tb_or_gate;

reg a;

reg b;

wire y;

or_gate dut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 0; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

$finish;

end

endmodule

### 5. Truth Table

### WAVEFORM

Tcl Console output:

a=0 b=0 y=0

a=0 b=1 y=1

a=1 b=0 y=1

a=1 b=1 y=1

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT2 with input buffer (IBUF) and output buffer (OBUF) is the expected implementation on the selected Spartan-7 FPGA, provided synthesis optimizes the logic into one LUT.

- LUT initialization value: INIT: 4'hE (as observed).

The LUT initialization value 4'hE corresponds to the binary configuration 4'b1110. For a two-input LUT, this configuration produces an output of 1 for input combinations 01, 10, and 11, and an output of 0 for 00, implementing the OR function.

## Gate 5 — NOR Gate Using OR + NOT

### 1. Objective

To design and verify a 2-input NOR gate by instantiating the previously designed OR and NOT gates using Verilog structural modeling.

### 2. Boolean Equation

Y = NOT (A OR B)

### 3. Verilog RTL — nor_gate.v

module nor_gate(

input a,

input b,

output y

);

wire o;

or_gate n1 (

.a(a),

.b(b),

.y(o)

);

not_gate n2 (

.a(o),

.y(y)

);

endmodule

### 4. Testbench — tb_nor_gate.v

`timescale 1ns / 1ps

module tb_nor_gate;

reg a;

reg b;

wire y;

nor_gate dut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 0; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

$finish;

end

endmodule

### 5. Truth Table

### WAVEFORM

Tcl Console output:

a=0 b=0 y=1

a=0 b=1 y=0

a=1 b=0 y=0

a=1 b=1 y=0

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT2 with input buffer (IBUF) and output buffer (OBUF) is the expected implementation on the selected Spartan-7 FPGA, provided synthesis optimizes the logic into one LUT.

- LUT initialization value: INIT: 4'h1 (as observed).

The LUT initialization value 4'h1 corresponds to the binary configuration 4'b0001. For a two-input LUT, this configuration produces an output of 1 only when both inputs are 0, implementing the NOR function.

## Gate 6 — XOR Gate Using Four NAND Gates

### 1. Objective

To design and verify a 2-input XOR gate by instantiating four previously designed NAND gates using Verilog structural modeling.

### 2. Boolean Equation

Y = A(B)' +  (A)'B

### 3. Verilog RTL — xor_gate.v

module xor_gate(

input a,

input b,

output y

);

wire n1;

wire n2;

wire n3;

nand_gate g1 (

.a(a),

.b(b),

.y(n1)

);

nand_gate g2 (

.a(a),

.b(n1),

.y(n2)

);

nand_gate g3 (

.a(b),

.b(n1),

.y(n3)

);

nand_gate g4 (

.a(n2),

.b(n3),

.y(y)

);

endmodule

### 4. Testbench — tb_xor_gate.v

`timescale 1ns / 1ps

module tb_xor_gate;

reg a;

reg b;

wire y;

xor_gate dut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 0; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

$finish;

end

endmodule

### 5. Truth Table

### WAVEFORM

Tcl Console output:

a=0 b=0 y=0

a=0 b=1 y=1

a=1 b=0 y=1

a=1 b=1 y=0

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT2 with input buffer (IBUF) and output buffer (OBUF) is the expected implementation on the selected Spartan-7 FPGA, provided synthesis optimizes the logic into one LUT.

- LUT initialization value: INIT: 4'h6 (as observed).

The LUT initialization value 4'h6 corresponds to the binary configuration 4'b0110. For a two-input LUT, this configuration produces an output of 1 when the inputs differ (01 or 10) and an output of 0 when the inputs are equal (00 or 11), implementing the XOR function.

## Gate 7 — XNOR Gate Using XOR + NOT

### 1. Objective

To design and verify a 2-input XNOR gate by instantiating the previously designed XOR and NOT gates using Verilog structural modeling.

### 2. Boolean Equation

Y = AB +  (A)'(B)'

### 3. Verilog RTL — xnor_gate.v

module xnor_gate(

input a,

input b,

output y

);

wire x;

xor_gate n1 (

.a(a),

.b(b),

.y(x)

);

not_gate n2 (

.a(x),

.y(y)

);

endmodule

### 4. Testbench — tb_xnor_gate.v

`timescale 1ns / 1ps

module tb_xnor_gate;

reg a;

reg b;

wire y;

xnor_gate dut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 0; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 0; #10;

$display("a=%b b=%b y=%b", a, b, y);

a = 1; b = 1; #10;

$display("a=%b b=%b y=%b", a, b, y);

$finish;

end

endmodule

### 5. Truth Table

### WAVEFORM

Tcl Console output:

a=0 b=0 y=1

a=0 b=1 y=0

a=1 b=0 y=0

a=1 b=1 y=1

### 6. Synthesis Observations

SCHEMATIC:

- Mapping: LUT2 with input buffer (IBUF) and output buffer (OBUF) is the expected implementation on the selected Spartan-7 FPGA, provided synthesis optimizes the logic into one LUT.

- LUT initialization value: INIT: 4'h9 (as observed).

The LUT initialization value 4'h9 corresponds to the binary configuration 4'b1001. For a two-input LUT, this configuration produces an output of 1 when both inputs are equal (00 or 11) and an output of 0 when the inputs differ (01 or 10), implementing the XNOR function.